# ZX Spectrum Companion Guide (Z80 Game Dev)

# 1. Memory Layout

- Screen bitmap: 16384–22527
- Attribute memory: 22528–23295
- ROM: 0–16383
- RAM: 16384 onwards

Screen is NOT linear — rows are interleaved (important for fast drawing).

# 2. Screen Addressing Formula

To calculate pixel address:

Address = 16384 + (Y & 0xC0)*32 + (Y & 0x07)*256 + (Y & 0x38)*4 + X

Where:
- X = column (0–31)
- Y = row (0–191)

# 3. Attributes

Each 8x8 block has one attribute byte:
Bits:
0–2: INK (foreground)
3–5: PAPER (background)
6: BRIGHT
7: FLASH

Example:
LD A, %01000111
LD (22528), A

# 4. Keyboard Scanning

Read keyboard via port FE:

LD BC, 0xFEFE
IN A,(C)

Bits represent keys (0 = pressed)

Typical rows:
FEFE: SHIFT,Z,X,C,V
FDFE: A,S,D,F,G

# 5. Simple Game Loop

Main loop:

LOOP:
  CALL READ_INPUT
  CALL UPDATE
  CALL DRAW
  JP LOOP

# 6. Sprite Drawing Idea

- Pre-store sprite bytes
- Use XOR drawing for simple erase/redraw
- Masking needed for proper overlap

# 7. Timing

Use delay loops or HALT:

HALT   ; waits for interrupt (~50Hz)

Good for syncing frames

# 8. Sound (Basic)

Use BEEP in ROM or toggle speaker bit manually via port FE.

# 9. Optimization Tips

- Use registers instead of memory
- Use DJNZ for loops
- Unroll loops where possible
- Precompute addresses