Retro ZX Spectrum Game Development
A quick crash course in Z80 assembly

Prepared as a practical primer for returning Spectrum developers.

| What this gives you | A practical Z80 refresher, tuned for game code, with examples and a compact instruction-set reference. |
| --- | --- |
| Best next step | Once this is familiar again, build tiny routines first: a sprite blit, a keyboard read, a timing loop, and a simple game loop. |

# 1. The Z80 in one page

The Z80 is an 8-bit CPU with a very approachable style: load data into registers, do arithmetic or logic, test the flags, and jump when the result tells you to. For Spectrum game work, the art lies in being economical with registers and ruthless about tight loops.

8-bit registers: A, B, C, D, E, H, L. The accumulator A is the star of the show.

16-bit register pairs: BC, DE, HL. Also SP (stack pointer), PC (program counter), IX and IY (index registers).

Flags live in F. The most commonly used are Z (zero) and C (carry).

Memory is byte-addressed. Instructions may use registers, immediates, or memory via (HL), (IX+d), or (IY+d).

Conditional flow is mostly built from JP, JR, CALL, RET, DJNZ, and tests on the flags.

A good mental model:

Z80 assembly is about moving bytes, shaping flags, and branching at exactly the right moment.

# 2. Registers and flags

| Item | Use in practice |
| --- | --- |
| A | Accumulator for arithmetic, logic, comparisons, and lots of data movement. |
| F | Flags register: S, Z, H, P/V, N, C, plus two undocumented copy bits. |
| BC / DE / HL | Common 16-bit working pairs. HL is the classic memory pointer. |
| IX / IY | Indexed pointers, handy but usually slower than HL. |
| SP | Stack pointer for CALL/RET, PUSH/POP, and temporary storage. |
| PC | Program counter; usually implicit. |
| AF' and EXX set | Alternate register bank for very fast context swaps. |

Flags worth burning into muscle memory:

| Flag | Meaning | Typical use |
| --- | --- | --- |
| Z | Zero | Loop termination, equality tests, JP Z / JR Z / RET Z |
| C | Carry | Unsigned overflow, 16-bit carries, JP C / JR C |
| S | Sign | Negative result in signed arithmetic |
| P/V | Parity / overflow | Signed overflow or parity, depending on instruction |
| N | Subtract flag | Set by subtraction/CP; useful for DAA context |
| H | Half carry | Mostly relevant for BCD with DAA |

# 3. Syntax and addressing

Common forms you will see again and again:

| Form | Example | Meaning |
| --- | --- | --- |
| r | LD A,B | 8-bit register |
| rr | LD HL,40000 | 16-bit register pair |
| n | LD A,42 | Immediate 8-bit value |
| nn | LD DE,49152 | Immediate 16-bit value |
| (HL) | LD A,(HL) | Memory pointed to by HL |
| (IX+d) | LD A,(IX+3) | Indexed memory with signed displacement |
| (IY+d) | BIT 0,(IY-1) | Same idea using IY |
| (nn) | LD A,(23560) | Absolute memory address |

Parentheses mean memory access: LD A,(16384) reads the byte stored at address 16384.

# 4. The core instruction families

You do not need every instruction in your head on day one. You do need the shape of the instruction set.

| Family | Key mnemonics | Why it matters |
| --- | --- | --- |
| Load / exchange | LD, PUSH, POP, EX, EXX | Moves data around and preserves state. |
| 8-bit arithmetic | ADD, ADC, SUB, SBC, INC, DEC, CP | Score logic, counters, comparisons, collision math. |
| Logic | AND, OR, XOR, CPL | Masking bits, toggles, fast clears, comparisons. |
| 16-bit arithmetic | ADD HL,rr / ADC HL,rr / SBC HL,rr | Pointer math and counters. |
| Flow control | JP, JR, DJNZ, CALL, RET, RST | Loops, branches, subroutines. |
| Bit ops | BIT, SET, RES | Game-state flags, keyboard masks, sprite attributes. |
| Rotate / shift | RLCA, RLA, RRCA, RRA, RLC, RRC, RL, RR, SLA, SRA, SRL | Bit scrolling, packed data, fast multiply/divide by powers of two. |
| I/O | IN, OUT | Useful when talking to hardware and peripherals. |
| Block ops | LDI, LDIR, CPI, CPIR, INI, OTIR, etc. | Fast repeated memory or port work. |
| Interrupt / CPU control | DI, EI, IM, NOP, HALT, DAA, SCF, CCF, NEG | System-level control and arithmetic edge cases. |

# 5. Tiny examples that bring it back

## A. Count down a loop

LD B,10          ; loop count

Loop:

; do something useful here

DJNZ Loop    ; decrement B and jump if not zero

DJNZ is gold for small loops. It is compact and quick.

## B. Compare and branch

LD A,(PlayerX)

CP 120           ; compare A with 120

JR C,StillLeft   ; jump if A < 120 (unsigned compare)

CP subtracts internally, sets flags, and throws the result away.

## C. Copy bytes with HL, DE, BC

LD HL,Source

LD DE,Dest

LD BC,32

CopyLoop:

LD A,(HL)

LD (DE),A

INC HL

INC DE

DEC BC

LD A,B

OR C

JR NZ,CopyLoop

This is the classic manual copy pattern. Later, you may switch to LDIR when appropriate.

## D. Toggle bits with XOR

LD A,(Flags)

XOR %00000100

LD (Flags),A

XOR is perfect for toggling individual bits when the mask is known.

# 6. Z80 habits that pay off in games

Prefer HL for hot memory pointers. IX and IY are convenient but not cheap.

Use A fearlessly. Many instructions naturally revolve around it.

Use DJNZ for short counted loops.

Keep an eye on what destroys flags. A beautiful compare is useless if you clobber Z or C before the branch.

PUSH and POP are useful, but stack abuse in an inner loop can get expensive.

EXX and EX AF,AF' are extremely handy when you want a fast temporary register swap.

Measure ideas against the Spectrum's video memory layout. Sometimes the data layout matters more than the instruction choice.

# 7. ZX Spectrum specifics worth remembering

The CPU is a Z80A running at roughly 3.5 MHz. On the 48K machine, the bitmap display starts at 16384 and attributes start at 22528. The bitmap is not laid out as a simple linear frame buffer; scan lines are interleaved in the famous Spectrum way. That means plotting and sprite work usually benefits from helper routines or precomputed tables.

Screen bitmap: 16384–22527

Colour attributes: 22528–23295

ROM routines can save time during experimentation, but many fast games eventually replace hot paths with custom code.

Interrupt timing and contention matter once you start pushing animation or music hard.

# 8. Prefixes: why some instructions look strange

The base opcode set is only part of the story. Prefix bytes extend or redirect instruction decoding.

| Prefix | Effect |
| --- | --- |
| CB | Bit, rotate, and shift instruction page |
| ED | Extended page: block ops, extra I/O, 16-bit ADC/SBC, NEG, RETI/RETN, IM |
| DD | Use IX in place of HL in many suitable instructions |
| FD | Use IY in place of HL in many suitable instructions |
| DD CB d / FD CB d | Indexed bit operations using displacement d |

This is why a full opcode map becomes large quickly: the instruction set is really a family of instruction pages.

# 9. Appendix A — condensed instruction-set reference

This appendix is a mnemonic/form reference. It is meant to be practical rather than byte-for-byte exhaustive. If you want, I can turn this into a full opcode matrix next.

## 8-bit load

Forms included

LD r,r'

LD r,n

LD r,(HL)

LD r,(IX+d)

LD r,(IY+d)

LD (HL),r

LD (IX+d),r

LD (IY+d),r

LD (HL),n

LD (IX+d),n

LD (IY+d),n

LD A,(BC)

LD A,(DE)

LD A,(nn)

LD (BC),A

LD (DE),A

LD (nn),A

LD A,I

LD A,R

LD I,A

LD R,A

## 16-bit load / stack / exchange

Forms included

LD rr,nn

LD HL,(nn)

LD rr,(nn) [ED page variants]

LD (nn),HL

LD (nn),rr [ED page variants]

LD SP,HL

LD SP,IX

LD SP,IY

PUSH qq

POP qq

EX DE,HL

EX AF,AF'

EXX

EX (SP),HL

EX (SP),IX

EX (SP),IY

## 8-bit arithmetic / compare

Forms included

ADD A,r

ADD A,n

ADD A,(HL)

ADD A,(IX+d)

ADD A,(IY+d)

ADC A,r

ADC A,n

ADC A,(HL)

ADC A,(IX+d)

ADC A,(IY+d)

SUB r

SUB n

SUB (HL)

SUB (IX+d)

SUB (IY+d)

SBC A,r

SBC A,n

SBC A,(HL)

SBC A,(IX+d)

SBC A,(IY+d)

INC r

INC (HL)

INC (IX+d)

INC (IY+d)

DEC r

DEC (HL)

DEC (IX+d)

DEC (IY+d)

CP r

CP n

CP (HL)

CP (IX+d)

CP (IY+d)

## Logic / misc accumulator

Forms included

AND r/n/(HL)/(IX+d)/(IY+d)

OR r/n/(HL)/(IX+d)/(IY+d)

XOR r/n/(HL)/(IX+d)/(IY+d)

CPL

DAA

SCF

CCF

NEG

## 16-bit arithmetic

Forms included

ADD HL,rr

ADD IX,pp

ADD IY,rr

INC rr

DEC rr

ADC HL,rr

SBC HL,rr

## Rotate / shift

Forms included

RLCA

RLA

RRCA

RRA

RLC r/(HL)/(IX+d)/(IY+d)

RL r/(HL)/(IX+d)/(IY+d)

RRC r/(HL)/(IX+d)/(IY+d)

RR r/(HL)/(IX+d)/(IY+d)

SLA r/(HL)/(IX+d)/(IY+d)

SRA r/(HL)/(IX+d)/(IY+d)

SRL r/(HL)/(IX+d)/(IY+d)

## Bit operations

Forms included

BIT b,r

BIT b,(HL)

BIT b,(IX+d)

BIT b,(IY+d)

SET b,r

SET b,(HL)

SET b,(IX+d)

SET b,(IY+d)

RES b,r

RES b,(HL)

RES b,(IX+d)

RES b,(IY+d)

## Flow control

Forms included

JP nn

JP cc,nn

JP (HL)

JP (IX)

JP (IY)

JR e

JR C,e

JR NC,e

JR Z,e

JR NZ,e

DJNZ e

CALL nn

CALL cc,nn

RET

RET cc

RETI

RETN

RST p

## Block transfer / search

Forms included

LDI

LDIR

LDD

LDDR

CPI

CPIR

CPD

CPDR

## Input / output

Forms included

IN A,(n)

IN r,(C)

INI

INIR

IND

INDR

OUT (n),A

OUT (C),r

OUTI

OTIR

OUTD

OTDR

## Interrupt / CPU control

Forms included

DI

EI

IM 0

IM 1

IM 2

NOP

HALT

# 10. Appendix B — what to practise first

Write a routine that fills 32 bytes of memory with a value.

Write a loop that animates a byte left and right using RLCA and RRCA.

Read a keyboard-related byte and test individual bits with BIT.

Build a sprite or character copy loop using HL, DE, and BC.

Write a compact delay loop and then replace it with frame-based timing.

If you want, the next document can be a ZX Spectrum-specific sequel: screen memory, attributes, keyboard scanning, interrupts, timing, and a tiny game loop skeleton.